# DESIGN DOCUMENTATION

# Automotive Multi-ECU CAN Communication System with Crash Detection Module

## 1. Introduction

Modern vehicles use multiple Electronic Control Units (ECUs) to control and monitor various automotive functions such as engine management, braking systems, infotainment, transmission control, battery monitoring, and safety systems. These ECUs communicate with each other using the Controller Area Network (CAN) protocol.

This project implements an Automotive Multi-ECU CAN Communication System with a Crash Detection Module using the STM32F407G Discovery Board. The crash detection module uses a KY-002 shock sensor to detect sudden vibration or impact conditions. When a crash event is detected, the STM32 microcontroller activates a warning LED and buzzer while transmitting a CAN message over the CAN bus using the TJA1050 CAN transceiver module.

The CAN communication is monitored using CANeasy software to simulate communication between multiple automotive ECUs.

# 2. Project Objective

The objectives of this project are:

- To simulate an automotive multi-ECU CAN communication network

- To detect crash events using the KY-002 shock sensor

- To process crash events using EXTI interrupt mechanism

- To activate LED and buzzer alerts during crash detection

- To transmit crash status through CAN communication

- To monitor CAN messages using CANeasy software

- To demonstrate embedded automotive communication architecture

# 3. System Overview

The project simulates a vehicle CAN network consisting of multiple ECUs.

| ECU Name | Function |
| --- | --- |
| Engine ECU | Engine parameter monitoring |
| ABS ECU | Brake system monitoring |
| Body Control Module (BCM) | Vehicle body electronics |
| Infotainment ECU | Display and multimedia system |
| Transmission ECU | Gear and transmission monitoring |
| Battery ECU | Battery monitoring system |
| Crash Detection ECU | Crash sensing and CAN transmission |

The Crash Detection ECU is implemented using the STM32F407G-DISC1 development board.

# 4. Hardware Components

| Sl No | Component | Purpose |
| --- | --- | --- |
| 1 | STM32F407G-DISC1 Discovery Board | Main microcontroller |
| 2 | KY-002 Shock Sensor | Crash and vibration detection |
| 3 | TJA1050 CAN Transceiver Module | CAN bus communication |
| 4 | LED | Crash indication |
| 5 | Buzzer | Audio alert |
| 6 | Jumper Wires | Hardware connections |
| 7 | Breadboard | Circuit prototyping |
| 8 | USB Cable | Power supply and programming |

# 5. Software Requirements

| Software | Purpose |
| --- | --- |
| STM32CubeIDE | Firmware development |
| CANeasy | CAN message monitoring |
| Embedded C | Firmware programming |
| STM32 HAL Library | Peripheral configuration |

# 6. Pin Configuration

| Component | STM32 Pin | Description |
| --- | --- | --- |
| KY-002 Shock Sensor | PA0 | EXTI interrupt input |
| LED | PD14 | Crash indication output |
| Buzzer | PA5 | Audio alert output |
| CAN TX | PD1 | CAN transmission |
| CAN RX | PD0 | CAN reception |

# 7. Working Principle

During normal operation, the KY-002 shock sensor continuously monitors vibration conditions. The sensor output is connected to PA0 configured as an EXTI interrupt input.

When vibration or sudden impact occurs:

KY-002 sensor detects vibration.

EXTI interrupt is triggered on PA0.

STM32 processes the crash event.

LED connected to PD14 starts blinking continuously.

Buzzer connected to PA5 activates for a few seconds.

CAN frame is transmitted using CAN ID 0x100.

CAN data byte 0x01 is sent indicating crash detection.

CANeasy monitors and displays the CAN message.

During normal condition:

CAN data byte 0x00 indicates normal status.

8. CAN Communication Design

## 8.1 CAN Configuration

| Parameter | Value |
| --- | --- |
| CAN Protocol | CAN 2.0 |
| CAN Transceiver | TJA1050 |
| CAN ID | 0x100 |
| Data Length | 1 Byte |
| Communication Type | Broadcast |
| CAN Speed | 500 kbps |

## 8.2 CAN Data Format

| CAN ID | Data Byte | Meaning |
| --- | --- | --- |
| 0x100 | 0x00 | Normal Condition |
| 0x100 | 0x01 | Crash Detected |

# 9. Finite State Machine (FSM)

The Crash Detection ECU operates using the following states:

| State | Description |
| --- | --- |
| IDLE | System initialization state |
| MONITORING | Monitoring sensor input |
| CRASH_DETECTED | Crash identified through EXTI interrupt |
| ALERT_TRANSMISSION | LED blinking, buzzer activation, and CAN transmission |

# 10. Block Diagram

# 11. Flowchart

START
   ↓
Initialize GPIO, CAN, EXTI
   ↓
Monitor KY-002 Sensor
   ↓
Crash Detected?
 ┌───────No────────┐
 ↓                 ↑
Yes                │
 ↓                 │
Trigger EXTI       │
 ↓                 │
Blink LED          │
 ↓                 │
Activate Buzzer    │
 ↓                 │
Transmit CAN Frame │
 ↓                 │
Return to Monitoring

# 12. Software Architecture

| File Name | Function |
| --- | --- |
| main.c | Main application |
| gpio.c | GPIO configuration |
| can.c | CAN communication |
| exti.c | External interrupt handling |
| crash_handler.c | Crash detection logic |

13. Testing and Validation

| Test ID | Test Description | Expected Result |
| --- | --- | --- |
| T1 | Sensor vibration test | EXTI interrupt triggered |
| T2 | LED test | LED blinks continuously |
| T3 | Buzzer test | Buzzer activates |
| T4 | CAN transmission test | CAN ID 0x100 transmitted |
| T5 | CAN data validation | 0x01 transmitted during crash |
| T6 | CANeasy monitoring | Message visible in CANeasy |

# 14. Advantages

Real-time crash detection

Fast CAN communication

Interrupt-based event handling

Simple embedded implementation

Automotive communication simulation

Low-cost hardware implementation

Internship-level automotive project

# 15. Future Scope

Real airbag deployment module integration

Multiple crash sensor support

GPS emergency notification

GSM-based accident alert system

Integration with additional ECUs

Advanced automotive diagnostics

CAN FD implementation

# 16. Conclusion

This project successfully demonstrates an Automotive Multi-ECU CAN Communication System with Crash Detection Module using the STM32F407G Discovery Board. The system detects vibration using the KY-002 shock sensor and processes crash events using EXTI interrupt handling. During crash detection, LED blinking and buzzer activation are performed while a CAN message is transmitted through the TJA1050 CAN transceiver.

The project demonstrates the fundamentals of automotive embedded systems, CAN communication, interrupt handling, and ECU interaction, making it suitable for internship-level automotive embedded applications.